GSPro Linear Actuator Wiring Guide

(Actuator with Built-In Limit Switches)

# Complete Parts List

### Electronics

- Arduino Uno R3

- BTS7960 43A Motor Driver Module

- 16V 10A Power Supply (for actuator via BTS7960)

- 12V Power Supply with barrel connector (for Arduino)

- USB Isolator Module

- USB A-to-B cable (6ft)

- 3x 1000µF 25V Electrolytic Capacitors

### Safety Components (Optional)

- 1x Emergency Stop Button (mushroom, NC, red) - Optional

- 3x LEDs (Red, Green, Yellow) - Optional but recommended

- 3x 220Ω resistors (for LEDs)

### Mounting & Wiring

- Project enclosure (6"x4"x3" minimum)

- Terminal blocks (screw type)

- 14AWG stranded wire (for long actuator run - red/black)

- 18AWG wire (for local power connections)

- 22AWG wire (for signals)

- Wire ferrules and heat shrink

- Cable glands for enclosure

- Rubber grommets or washers (for vibration dampening)

# Power Distribution

16V 10A Power Supply (Actuator Power)

└─→ 3x 1000µF Capacitors (parallel at B+/B-)

└─→ BTS7960 B+ and B-

└─→ 14AWG twisted pair (35ft) → Linear Actuator

12V Power Supply (Arduino Power)

└─→ Arduino barrel jack connector

PC USB Port

└─→ USB Isolator Module

└─→ Arduino USB port (data only)

Star Ground Connection:

├─→ 16V Power Supply GND

├─→ 12V Power Supply GND

├─→ BTS7960 B-

└─→ Arduino GND

Important Notes:

- Two separate power supplies: 16V for actuator, 12V for Arduino

- All grounds connected at a single star ground point

- USB Isolator between PC and Arduino for noise isolation

- Use 14AWG stranded twisted pair for long actuator wire run

- 3x 1000µF capacitors at BTS7960 B+/B- for EMI suppression

- Add inline fuse (10A) on 16V line for safety

# Arduino Pin Connections

### Motor Driver (BTS7960)

| BTS7960 Pin | Arduino Pin | Description |
| --- | --- | --- |
| RPWM | 5 | Right PWM (retract = RH) |
| LPWM | 6 | Left PWM (extend = LH) |
| R_EN | 7 | Right Enable |
| L_EN | 8 | Left Enable |
| VCC | 5V | Logic power |
| GND | GND | Common ground (star ground) |

### BTS7960 Power Connections

| BTS7960 Pin | Connection |
| --- | --- |
| B+ | 16V Power Supply + (through 3x 1000µF caps) |
| B- | 16V Power Supply - (star ground) |
| M+ | 14AWG to Actuator wire 1 (red) |
| M- | 14AWG to Actuator wire 2 (black) |

### Safety Switch (Optional)

| Component | Arduino Pin | Notes |
| --- | --- | --- |
| E-Stop Button (NC) | Pin 4 | Use INPUT_PULLUP, optional |

### Status LEDs (Optional - with 220Ω resistors)

| LED Color | Arduino Pin | Meaning |
| --- | --- | --- |
| Red | Pin 10 | Left-Handed Position |
| Green | Pin 11 | Right-Handed Position |
| Yellow | Pin 12 | Moving |

LED Wiring: Arduino Pin → 220Ω Resistor → LED Anode (+) → LED Cathode (-) → GND

# Long Actuator Wire Run

This configuration places the Arduino, BTS7960, and power supplies near the PC, with a long wire run to the remotely-mounted actuator. This keeps EMI-generating components away from sensitive USB communication.

### Wire Specifications

| Parameter | Specification |
| --- | --- |
| Wire Gauge | 14 AWG stranded copper |
| Wire Length | 35 feet (each conductor) |
| Configuration | Twisted pair (1-2 twists per inch) |
| Voltage Drop | ~1.1V at 5A (actuator sees ~14.9V) |
| Current Rating | 5A maximum (actuator spec) |

### Best Practices for Long Wire Run

- Twist the positive and negative wires together for the entire run

- Keep both conductors bundled - do not run on separate paths

- Add a 100-1000µF capacitor at the actuator end for local energy storage

- Optional: ferrite clamp at actuator end to suppress motor brush noise

- Use quality terminal connections or solder joints at both ends

# USB Isolator Installation

The USB isolator provides galvanic isolation between the PC and Arduino, preventing electrical noise from the motor circuit from disrupting USB communication.

PC USB Port

│

└─→ [USB Isolator Module] ←── Install at Arduino end

│

└─→ Arduino USB Port

Installation Notes:

- Install isolator at the Arduino end of the USB cable

- Keep USB cable as short as practical (6ft recommended)

- USB provides data only - Arduino powered separately via 12V barrel jack

# Actuator Built-In Limit Switches

Your actuator has internal limit switches that automatically:

- Stop extension at full stroke (28"/30")

- Stop retraction at 0"

- Prevent damage from over-travel

- Cut power when limits are reached

No external switches needed! The Arduino uses timed movement:

- Calculates expected movement time (~21 seconds for 28")

- Stops sending power after movement completes

- Built-in limits provide hardware backup

# Emergency Stop Wiring (Optional)

Arduino Pin 4 ──── [E-Stop Button (NC)] ──── GND

When E-stop pressed: button opens, pin floats HIGH (via pullup) → motor stops

Normal operation: button closed, pin LOW

Note: E-stop is optional since actuator has built-in protection. However, it is good practice for any automated system.

# Assembly Steps

### 1. Prepare Enclosure

Drill holes for:

- USB cable gland

- 12V Arduino power input

- 16V BTS power input

- Actuator output (2-wire connection for 14AWG)

- E-stop button mounting (optional)

- Status LEDs (front panel, optional)

### 2. Mount Components Inside Enclosure

- Arduino on standoffs

- BTS7960 driver on standoffs (ensure good airflow)

- 3x 1000µF capacitors near BTS7960 B+/B- terminals

- Terminal blocks for connections

### 3. Power Wiring (POWER OFF!)

16V Supply + ──[FUSE 10A]──┬─→ 1000µF caps (+) ─→ BTS7960 B+

│

16V Supply - ──────────────┼─→ 1000µF caps (-) ─→ BTS7960 B-

│

└─→ Star Ground ←─┬─ 12V Supply -

└─ Arduino GND

12V Supply + ─────────────────→ Arduino barrel jack center (+)

12V Supply - ─────────────────→ Star Ground

### 4. Signal Wiring

- Connect Arduino to BTS7960 per pin table above

- Connect E-stop to pin 4 (if using)

- Connect LEDs to pins 10, 11, 12 (if using)

### 5. Actuator Connection

- BTS7960 M+ → 14AWG twisted pair → Actuator red wire

- BTS7960 M- → 14AWG twisted pair → Actuator black wire

- Optional: Add 100-1000µF cap across actuator terminals at actuator end

### 6. USB Connection

- PC USB port → USB Isolator → Arduino USB port

# Vibration Dampening (Actuator Mount)

To reduce acoustic noise transmitted through the mounting structure:

- Use rubber grommets or washers at each mounting screw

- Place rubber washers on both sides of the mounting plate

- Optional: thin rubber pad between mounting plate and beam

- Rubber grommets are more durable than foam for long-term use

# Testing Procedure

### Phase 1: Power-On Test (No Actuator)

- Connect 16V power to BTS7960

- Connect 12V power to Arduino barrel jack

- Arduino power LED should light

- Connect USB through isolator to PC

- Upload Arduino code

- Open Serial Monitor (9600 baud)

- Should see "GSPro Actuator Controller Ready"

### Phase 2: Manual Control Test (No Actuator)

- Type "STATUS" in Serial Monitor → verify response

- Status LEDs should indicate current state (if installed)

- Press E-stop (if installed) → LEDs should flash

- Release E-stop → normal operation

### Phase 3: Actuator Test (REDUCED SPEED)

⚠️ Reduce MOVE_SPEED in code to 128 (50% speed) for initial testing

- Connect actuator via 14AWG twisted pair

- Ensure clear travel path - nothing in the way!

- Actuator should start in retracted position (RH)

- Send "LH" command → actuator extends slowly

- Watch it extend - built-in limit will stop it

- Send "RH" command → actuator retracts slowly

- Verify E-stop immediately stops motion (if installed)

- Verify USB connection remains stable during movement

### Phase 4: Full Speed Test

- Restore MOVE_SPEED to 255 in code

- Upload updated code

- Send "RH" to ensure starting position

- Send "LH" command and time the movement

- Send "RH" command and time the return

- Adjust MOVE_TIME constants if timing is off

### Phase 5: Python Integration Test

- Ensure: pip install pyserial

- Run Python script

- Should auto-detect Arduino

- Change player handedness in GSPro

- Verify actuator moves automatically

- Test multiple switches back and forth

# Calibration & Fine-Tuning

### Speed Adjustment

- Modify MOVE_SPEED (0-255) in Arduino code

- Lower values = slower, quieter

- Higher values = faster movement

- Recommend 200-255 for normal use

- Note: 16V supply provides ~33% more speed than 12V

### Timing Calibration

After testing, measure actual movement times:

- Time LH to RH movement

- Time RH to LH movement

- Update MOVE_TIME_LH_TO_RH and MOVE_TIME_RH_TO_LH in code

- Set MOVE_BUFFER (safety buffer, 2-3 seconds)

### Direction Correction

If actuator moves wrong direction:

- Option 1: Swap M+ and M- wires at BTS7960

- Option 2: In Arduino code, swap moveLeft() and moveRight() functions

# Troubleshooting

### Actuator doesn't move

- Check 16V power supply with multimeter

- Verify BTS7960 connections

- Test M+ and M- voltage when moving (should show ~16V)

- Check Arduino pins 7 & 8 (enable pins) are going HIGH

- Verify RPWM/LPWM pins are outputting PWM

- Check 14AWG connections at both ends

### USB disconnects during movement

- Verify USB isolator is installed at Arduino end

- Check star ground connections are secure

- Verify 1000µF capacitors are properly connected at BTS7960

- Ensure actuator wires are twisted pair

- Add ferrite clamp to actuator wires at actuator end

### Moves wrong direction

- Swap M+ and M- wires on BTS7960

- Or swap RPWM/LPWM functionality in code

### Movement time is wrong

- Measure actual time with stopwatch

- Update MOVE_TIME constants in code

- Note: 16V will be faster than original 12V timing

- Account for load - movement may be slightly slower with weight

### Python can't find Arduino

- Check USB cable is data-capable (not charge-only)

- Verify USB isolator is working (LEDs if present)

- Check Device Manager (Windows) for COM port

- Install Arduino drivers if needed

- Manually specify port in Python code if auto-detect fails

### Erratic movement or buzzing

- Reduce MOVE_SPEED

- Verify 1000µF capacitors are connected correctly

- Check all ground connections are secure at star ground

- Ensure 16V power supply can deliver 10A

# Safety Notes

⚠️ IMPORTANT SAFETY WARNINGS:

- Clear travel path before automated operation

- Built-in limits are primary protection

- Keep hands clear during automated movement

- Test at reduced speed first

- Use proper wire gauge for current capacity (14AWG for long run)

- Secure all connections with ferrules or solder

- Mount enclosure securely

- Label all wiring clearly

- E-stop provides additional safety layer

- Disconnect BOTH power supplies when working on system

- 16V exceeds actuator rating - monitor for overheating

# System Advantages

✓ Stable USB connection - electronics near PC, long wire to actuator

✓ EMI isolation - USB isolator + capacitors + twisted pair

✓ Faster actuator speed - 16V supply vs 12V rated

✓ Simpler wiring - No external limit switch installation

✓ More reliable - Factory-calibrated limit positions

✓ Independent power - Separate supplies for logic and motor

✓ Vibration dampened - Rubber grommets at actuator mount

# Recommended Improvements

### Optional Enhancements

- Current Sensing - Detect obstructions or jams

- HC-05 Bluetooth Module - Wireless control alternative

- OLED Display - Show status without PC

- Buzzer - Audio feedback for movements

- Manual Control Pendant - Wired remote with RH/LH buttons

- Soft Start/Stop - Acceleration ramps for smoother motion

### Future Upgrades

- Mobile app control via Bluetooth

- Integration with other sim equipment

- Voice control

- Data logging (movements, errors, usage hours)

- Temperature monitoring for overvolted actuator